# Automated cropping and scale bars in imageJ

Setting the Global Scale

First set the global scale using a 10X image of the Neubauer chamber. This should be every time before running macros.

- Open the Reference Image:

  - Start Fiji and open the image of the Neubauer chamber (or any other reference image you use to set the scale).

  - Use File > Open and navigate to the location of your reference image.

- Draw the Line for Scale:

  - Select the straight line tool from the toolbar.

  - Draw a line across a known distance (e.g., spanning 4 squares of the Neubauer chamber which equals 1mm/1000µm).

- Set the Scale:

  - Go to Analyze > Set Scale.

  - In the Set Scale window, enter the known distance in microns (1000 µm if the line represents 1mm) in the Known Distance field.

  - Enter the unit of length (µm).

  - Make sure to check the Global checkbox to apply this scale to all images you open in this session.

  - Click OK to set the scale.

- Save the Settings (Optional):

  - You might want to save these settings for future use.

  - Go to File > Save As and choose an appropriate format to save your reference image with the scale set.

Running the Macros

- Open the Macro for Opening Images:

  - Plugins > macros > edit > open the “open_pics.ijm” macro. Edit this to the location of the photos. Run the macro, and all images should open.

- Open the Macro for Cropping and Adding Scale Bars:

  - Once all images are open, open the “crop_scale_bar.ijm” macro in the same way

  - Do any editing of crop size and/or scale bar options.

  - Choose a folder to save the edited photos in.

  - Before running the macro, ensure all images are open and the global scale is set.

  - Run this macro by clicking Run. It will prompt you to select the cropping area on each image (use rectangle tool and draw a rectangle in the center of the spheroid, see picture below), then crop the image, add a scale bar, and save it.

- Save Cropped Images:

  - The cropped images with the scale bar will be automatically saved in the specified directory.

  - Make sure you have the correct path set in the outputDirectory variable of your macro.

Closing Remarks

- Remember to close any images you don't need before starting a new macro to avoid confusion.

- Each time you start a new Fiji session, repeat the global scale-setting process with the reference image to ensure accuracy.

- The cropping size and scale bar size are set in the macro. Adjust these values if your requirements change.

- For batch processing, ensure that the file extensions in the macro match the image files in your directory.